CLI Command List

CLI Command List	1

Introduction	2

1. Status	3

1.1.	Basic Network	3

1.1.1.	WAN & Uplink	3

1.1.2.	LAN 	4

1.2.	DDNS	4

2.	Basic Network	5

2.1.	WAN & Uplink	5

2.1.1.	physical interface	5

2.2.	Port Forwarding	8

2.2.1.	Remote Administrator	8

2.3.	DDNS	9

2.3.1.	Configuration	9

2.4.	System Operation	10

2.4.1.	Password & MMI	10

2.4.2.	System Information	11

2.4.3.	Reboot & Reset	12

2.4.4.	Language	12

2.4.5.	Schedule	12

3.	Administration	14

3.1.	Configure & Manage	14

3.1.1.	SSH	14

3.1.2.	AT_NMEA	14

Introduction

The CLI Command formation is in following, but there still have a few exceptions.

| Entity | Instance | Parameter | Values | Description |
| --- | --- | --- | --- | --- |

And some of them also need extracommand “apply” to take effect

- Status

  - Basic Network

    - WAN & Uplink

WAN Interface IPv4 Network Status

| Entity | Instance | Parameter | Values | Description |
| --- | --- | --- | --- | --- |
| status | 1 | wiface_ipv4 | ?? | n: 3g index 1 |

WAN Interface IPv6 Network Status

| Entity | Instance | Parameter | Values | Description |
| --- | --- | --- | --- | --- |
| status | 0 | wiface_ipv6 | ?? |  |

LAN Interface Network Status

| Entity | Instance | Parameter | Values | Description |
| --- | --- | --- | --- | --- |
| status | 0 | liface | ?? |  |

3G/4G Modem Status List

| Entity | Instance | Parameter | Values | Description |
| --- | --- | --- | --- | --- |
| status | 0 | 3g | ?? |  |
| status | 1 | 3g_modem | ?? | n: 3g index 1 |
| status | 1 | 3g_sim | ?? | n: 3g index 1 |
| status | 1 | 3g_service | ?? | n: 3g index 1 |

    - LAN

LAN Client List

| Entity | Instance | Parameter | Values | Description |
| --- | --- | --- | --- | --- |
| status | 0 | lan_client | ?? |  |

  - DDNS

DDNS Status

| Entity | Instance | Parameter | Values | Description |
| --- | --- | --- | --- | --- |
| status | 0 | ddns_status | ?? |  |

- Basic Network

  - WAN & Uplink

    - physical interface

| Entity | Instance | Parameter | Values | Description |
| --- | --- | --- | --- | --- |
| eth | 0 | device_mode | 0/1 | 0: NAT 1: Modem |

| apply | eth |  |  |  |
| --- | --- | --- | --- | --- |

Lan

| Entity | Instance | Parameter | Values | Description |
| --- | --- | --- | --- | --- |
| eth | 21 | ipaddr | Ip |  |
| eth | 21 | mask | Ip |  |

| apply | dhcp |  |  |  |
| --- | --- | --- | --- | --- |

connection

| sim | 1 | profile_a | 0/1 | 0: auto-detection 1: Manual-configuration |  |
| --- | --- | --- | --- | --- | --- |
| sim | 1 | ip_type_a | 0/1/2 | 0: IPv4 1: IPv6 2:IPv4/6 |  |
| sim | 1 | pin_code_a | pin_code |  |  |

Manual-configuration

| Entity | Instance | Parameter | Values | Description |
| --- | --- | --- | --- | --- |
| sim | 1 | apn_a | str_apn |  |
| sim | 1 | username_a | name | 3g/4g |
| sim | 1 | password_a | passwd | 3g/4g |
| sim | 1 | auth_a | 0/1/2 | 0: auto 1: pap 2: chap |

IP mode

| Entity | Instance | Parameter | Values | Description |
| --- | --- | --- | --- | --- |
| sim | 1 | ip_mode_a | 0/1 | 0:Dynamic IP 1:Static IP |
| sim | 1 | ip_a | IP Address | Ex:10.10.10.1 |
| sim | 1 | mask_a | IP Address | Ex:255.255.255.0 |
| sim | 1 | gateway_a | IP Address | Ex:10.10.10.2 |
| sim | 1 | dns1_a | IP Address | Ex:168.95.1.1 |
| sim | 1 | dns2_a | IP Address | Ex:8.8.8.8 |

MTU

| Entity | Instance | Parameter | Values | Description |
| --- | --- | --- | --- | --- |
| sim | 1 | mtu | 0,68~1500 | 0(disable) |

Keep Alive

| Entity | Instance | Parameter | Values | Description |
| --- | --- | --- | --- | --- |
| sim | 1 | keep_alive | on/off | on:enable off:disable |
| sim | 1 | ka_ip | IP Address | Ex:8.8.8.8 |
| sim | 1 | ka_Interval | 2~14400 | unit:sec |

SIM Select

| Entity | Instance | Parameter | Values | Description |
| --- | --- | --- | --- | --- |
| sim | 0 | sel_mode | 0/1/2 | 0:auto 1:external 2:internal |

| apply | sim |  |  |  |
| --- | --- | --- | --- | --- |

  - Port Forwarding

    - Remote Adaminstrator

Configuration

| Entity | Instance | Parameter | Values | Description |
| --- | --- | --- | --- | --- |
| nb | 0 | conf_vs_enable | on/off | off: Disable on: Enable |
| apply | vs |  |  |  |

List

| Entity | Instance | Parameter | Values | Description |
| --- | --- | --- | --- | --- |
| nb | 0 | vs_list | ?? |  |
| nb | 1 | vs_public_port |  | specificport: x port range: x-x |
| nb | 1 | vs_name |  |  |
| nb | 1 | vs_server_ip |  |  |
| nb | 1 | vs_private_port |  | specificport: x port range: x-x |
| nb | 1 | vs_protocol | 0/1/2/3 | 0: TCP(6) & UDP(17) 1: TCP 2: UDP 3:Other |
| nb | 1 | vs_protocol_other |  |  |
| nb | 1 | vs_enable | on/off | off: Disable on: Enable |
| nb | 0 | vs_del | n |  |

| apply | vs |  |  |  |
| --- | --- | --- | --- | --- |

  - DDNS

    - Configuration

Dynamic DNS

| Entity | Instance | Parameter | Values | Description |
| --- | --- | --- | --- | --- |
| dydns | 0 | enable | on/off |  |
| dydns | 0 | provider | 0/1/2/ | 0: DynDNS.org (Dynamic) 1: No-IP.com 2: TZO.com |
| dydns | 0 | hostname |  |  |
| dydns | 0 | username |  |  |
| dydns | 0 | password |  |  |

| apply | ddns |  |  |  |
| --- | --- | --- | --- | --- |

  - System Operation

    - Password & MMI

Password

| Entity | Instance | Parameter | Values | Description |
| --- | --- | --- | --- | --- |
| sysrelated | 0 | change_pwd | oldpassword | Enter the old password before change to the new password. The maximum length of the passwordis 9. |

MMI

| Entity | Instance | Parameter | Values | Description |
| --- | --- | --- | --- | --- |
| mmi | 0 | ui_loginpwdatkmax | 3 ~ 10 |  |
| mmi | 0 | ui_admintimeouten | on/off | off: Disable on: Enable |
| mmi | 0 | ui_admintimeout | 30 ~ 65535 | sec |

    - System Information

| Entity | Instance | Parameter | Values | Description |
| --- | --- | --- | --- | --- |
| sysrelated | 0 | fw_version | ?? |  |
| sysrelated | 0 | fw_date | ?? |  |
| sysrelated | 0 | model_name | ?? |  |
| sysrelated | 0 | device_serial_number | ?? |  |
| sysrelated | 0 | sys_time | ?? |  |
| sysrelated | 0 | up_time | ?? |  |

Time Server

| Entity | Instance | Parameter | Values | Description |
| --- | --- | --- | --- | --- |
| sysrelated | 0 | time_zone |  | enter command “ sysrelated 0 time_zone” can query time zone number |
| sysrelated | 0 | time_server |  |  |
| sysrelated | 0 | time_daylight_en | on/off | off: Disable on: Enable |
| sysrelated | 0 | time_daylight_shour | 0~23 |  |
| sysrelated | 0 | time_daylight_sday | 1~31 |  |
| sysrelated | 0 | time_daylight_smonth | 1~12 |  |
| sysrelated | 0 | time_daylight_ehour | 0~23 |  |
| sysrelated | 0 | time_daylight_eday | 1~31 |  |
| sysrelated | 0 | time_daylight_emonth | 1~12 |  |
| apply | time |  |  |  |

| sysrelated | 0 | time_syncwithserver | 1 | Synchronize immediately |
| --- | --- | --- | --- | --- |

    - Reboot & Reset

| Entity | Instance | Parameter | Values | Description |
| --- | --- | --- | --- | --- |
| reboot |  |  |  |  |
| factory_reset |  |  |  |  |

    - Language

| Entity | Instance | Parameter | Values | Description |
| --- | --- | --- | --- | --- |
| sysrelated | 0 | set_language | 1-5 | 1 or others : English 5 : Japanese |

    - Schedule

Configuration

| Entity | Instance | Parameter | Values | Description |
| --- | --- | --- | --- | --- |
| schedule | 1 | schedule_name | name |  |
| schedule | 1 | schedule_policy | 0/1 | 0: Inactivate 1:activate |
| schedule | 1 | schedule_wday1 | 1/2/4/8/16/32/64/127 | 1: Sunday 2: Monday 4: Tuesday 8: Wednesday 16: Thursday 32: Friday 64: Saturday 127: Every Day |
| schedule | 1 | schedule_stime1 | hh:mm |  |
| schedule | 1 | schedule_etime1 | hh:mm |  |
| schedule | 0 | schedule_list | ?? |  |
| schedule | 1 | schedule_del | 0 | n: Time Schedule ID |

| apply | schedule |  |  |  |
| --- | --- | --- | --- | --- |

- Administration

  - Configure & Manage

    - SSH

Configuration

| Entity | Instance | Parameter | Values | Description |
| --- | --- | --- | --- | --- |
| sysmgmt | 0 | ssh_lan_en | on/off |  |
| sysmgmt | 0 | ssh_wan_en | on/off |  |
| sysmgmt | 0 | ssh_port |  | 1~65535 |

| apply | telnet_ssh |  |  |  |
| --- | --- | --- | --- | --- |

  - AT&NMEA

| Entity | Instance | Parameter | Values | Description |
| --- | --- | --- | --- | --- |
| at_nmea_conf |  |  | on/off |  |

- Example

  - CLI thru SSH with Python

Purpose: Without webGUI accessing to get the related MDG100 information on WInPC/LinuxPC at LAN side.

Steps : Create a MDG100_ssh_input.py and give a password argument.

Ex: python MDG100_ssh_input.py web_password

MDG100_ssh_input.py code as below:

import paramiko

import sys

if __name__ == '__main__':

if len(sys.argv) < 2:

print('no password input')

sys.exit()

username = "admin"

password = sys.argv[1]

hostname = "172.16.0.1"

port = 22

try:

client = paramiko.SSHClient()

client.set_missing_host_key_policy(paramiko.AutoAddPolicy())

client.connect(hostname, port, username, password)

client.exec_command('cd /tmp')

print ("==========DHCP Client=========")

stdin, stdout, stderr = client.exec_command('status 0 lan_client ??')

result = stdout.readlines()

print (result)

print ("==========Mode================")

stdin, stdout, stderr = client.exec_command('eth 0 device_mode ??')

result = stdout.readlines()

print (result)

print ("0:NAT 1:bridge")

print ("=======SIM select==============")

stdin, stdout, stderr = client.exec_command('sim 0 sel_mode ??')

result = stdout.readlines()

print (result)

print ("SIM select 0:auto 1:external 2:internal")

print ("========== FW date=============")

stdin, stdout, stderr = client.exec_command('sysrelated 0 fw_date ??')

result = stdout.readlines()

print (result)

print ("========== FW Version==========")

stdin, stdout, stderr = client.exec_command('sysrelated 0 fw_version ??')

result = stdout.readlines()

print (result)

print ("=======Cellular status=========")

stdin, stdout, stderr = client.exec_command('status 0 3g ??')

result = stdout.readlines()

print (result)

print ("=======Cellular FW=============")

stdin, stdout, stderr = client.exec_command('status 1 3g_modem ??')

result = stdout.readlines()

print (result)

print ("=======SIM status==============")

stdin, stdout, stderr = client.exec_command('status 1 3g_sim ??')

result = stdout.readlines()

print (result)

print ("=======APN name================")

stdin, stdout, stderr = client.exec_command('sim 1 apn_a ??')

result = stdout.readlines()

print (result)

print ("=======APN profile=============")

stdin, stdout, stderr = client.exec_command('sim 1 profile_a ??')

result = stdout.readlines()

print ("0:auto 1:manual")

print (result)

print ("=======Cellular Signal=========")

stdin, stdout, stderr = client.exec_command('status 1 3g_service ??')

result = stdout.readlines()

print (result)

print ("=======WAN state================")

stdin, stdout, stderr = client.exec_command('status 1 wiface_ipv4 ??')

result = stdout.readlines()

print (result)

print ("=======LAN state===============")

stdin, stdout, stderr = client.exec_command('status 0 liface ??')

result = stdout.readlines()

print (result)

except Exception:

print ('Exception!!')

raise

  - Results

==========DHCP Client=========

['  IP Address: 172.16.0.7\n', '  Host Name: AMO\n', '  MAC Address: 7E:F2:2F:CF:7B:F7\n', '  Remaining Lease Time: 1662458054 sec.\n']

==========Mode================

['0\n']

0:NAT 1:bridge

=======SIM select==============

['1\n']

SIM select 0:auto 1:external 2:internal

========== FW date=============

['2022/07/20\n']

========== FW Version==========

['00004D0.M61_e61.0000_07201200\n']

=======Cellular status=========

['  Card Info: EC25\n', '  Link Status: Connected\n', '  Signal Strength: 87\n', '  Network Name: Chunghwa Telecom\n']

=======Cellular FW=============

['Module Name: EC25\n', 'IMEI/MEID: 865546046045627\n', 'FW Version: EC25EUGAR06A01M4G\n', 'Temprature: 42\n', 'Band Capability: B1,B3,B7,B8,B20,B28,B38,B40,B41\n']

=======SIM status==============

['PIN Code Status: Ready\n', 'PIN Code Remaining Times: 3\n', 'PUK Code Remaining Times: 10\n']

=======APN name================

['\n']

=======APN profile=============

0:auto 1:manual

['1\n']

=======Cellular Signal=========

['Operator: Chunghwa Telecom\n', 'IMSI: 466924253070800\n', 'MCC: 466\n', 'MNC: 92\n', 'LAC: N/A\n', 'TAC: 8E30\n', 'Cell ID: 960DD0D\n', 'Service Type: LTE\n', 'Band: Band 3\n', 'CS Register Status: Registered\n', 'Roaming Status: Not Roaming\n', 'RSSI: 59\n', 'RSRP: 91\n', 'RSRQ: 8\n', 'RSCP: N/A\n', 'EcIo: N/A\n', 'SINR: 15\n', 'ICCID: 89886920042530708002\n']

=======WAN state================

['  IP: 10.222.26.139\n', '  Subnet Mask: 255.255.255.248\n', '  Gateway: 10.222.26.140\n', '  dns1: 168.95.1.1\n', '  dns2: 168.95.192.1\n', 'Connect Status: Connected\n']

=======LAN state===============

['  IPv4 Address: 172.16.0.1\n', '  IPv4 Subnet Mask: 255.255.255.0\n', '  IPv6 Global IP Address: /64\n', '  MAC Address: 9A:5E:73:33:62:63\n']

  - AT commands sample

username = "admin"

password = sys.argv[1]

hostname = "172.16.0.1"

port = 22

try:

client = paramiko.SSHClient()

client.set_missing_host_key_policy(paramiko.AutoAddPolicy())

client.connect(hostname, port, username, password)

client.exec_command('cd /tmp')

print ("==========cpin==========")

stdin, stdout, stderr = client.exec_command('sendATcmd  -c at+cpin?')

result = stdout.readlines()

print (result)

print ("=======cgcontrdp========")

stdin, stdout, stderr = client.exec_command('sendATcmd  -c at+cgcontrdp')

result = stdout.readlines()

print (result)

print ("========cgatt===========")

stdin, stdout, stderr = client.exec_command('sendATcmd  -c at+cgatt?')

result = stdout.readlines()

print (result)

print ("========cgact===========")

stdin, stdout, stderr = client.exec_command('sendATcmd  -c at+cgact?')

result = stdout.readlines()

print (result)

print ("========qnwinfo=========")

stdin, stdout, stderr = client.exec_command('sendATcmd  -c at+qnwinfo')

result = stdout.readlines()

print (result)

print ("=========cops===========")

stdin, stdout, stderr = client.exec_command('sendATcmd  -c at+cops?')

result = stdout.readlines()

print (result)

print ("==========cgreg=========")

stdin, stdout, stderr = client.exec_command('sendATcmd  -c at+cgreg?')

result = stdout.readlines()

print (result)

print ("=========creg===========")

stdin, stdout, stderr = client.exec_command('sendATcmd  -c at+creg?')

result = stdout.readlines()

print (result)

except Exception:

print ('Exception!!')

raise